Arburg Technology Days 2026

Innovative solutions for silicone processing

Variable: Production of different puzzle pieces with 
5-cavity family mould

Fully automated: Production of medical LSR closures as mass-produced items

Flexible: Silicone mould inserts for small quantities

Lossburg, 10/03/2026

At the Technology Days 2026, Arburg and its renowned partners will demonstrate the versatility of silicone in injection moulding and the technical innovations that enable process reliability. Examples of products manufactured include an LSR puzzle, LSR closures as mass-produced items and individual puzzle pieces.

As a pioneer, Arburg has always been involved in new developments in LSR injection moulding and has in-depth process expertise. Current process technology highlights include a cold runner measurement and control system from Elmet integrated into the Gestica control system as well as flow and temperature control with the “Arburg FlowPilot”.

LSR puzzle: Peripheral equipment and robot are integrated

A five-piece puzzle demonstrates the expertise of Arburg and its partner Elmet in LSR processing and mould technology. An electric Allrounder 520 A produces five different sizes of puzzle pieces in a cycle time of around 60 seconds using a sophisticated multi-cavity mould. Automatic, precise balancing of the five mould cavities is assured by the new “SMARTshot i” cold runner measurement and control concept with servo-electric needle-type shut-off nozzles and dynamically adjustable needle stroke. During injection, the thrust forces on then needles are recorded and transmitted to the Gestica control system via the Euromap EM 75 interface. This new system enables simultaneous, fully automatic filling of mould cavities with different material volumes. In addition, rheological fluctuations are detected automatically without the need for an additional sensor system. This can significantly reduce the process start-up time.

The associated dosing system is integrated into the Gestica via EM 82.3. A Yaskawa six-axis robot, which is also integrated into the control system, removes the components and performs the set-down operation on two conveyor belts. These individual components are then transported away in five containers.

Arburg FlowPilot: Temperature control of cold runner moulds

For a controlled process and high part quality in LSR processing, the “Arburg FlowPilot” can be used to regulate the flow and temperature for each cold runner individually. The “Arburg FlowPilot” can be combined with temperature control units from various manufacturers and is available with either a 4-way or 8-way manifold. The processes are actively monitored by OPC UA. In the event of malfunctions or machine standstill, its channels open automatically. This ensures full flow and prevents unwanted interlinking of the LSR components in the mould. At the Technology Days 2026, this will be demonstrated during the production of LSR bottle teats on an electric Allrounder 470 A with an 8-cavity mould from Rico.

The “Arburg FlowPilot” is also available for processing thermoplastics with hot runner moulds. Depending on the application and thanks to adjusted pump speed and optimum utilisation of the mould’s cooling capacity, the energy requirement of the temperature control units can be reduced by over 85 per cent and the cycle time by up to 75 per cent.

64 LSR closures in 25 seconds

An electric Allrounder 1800 e Trend demonstrates that Arburg’s new electric standard machines are also suitable for the fully automated production of standard LSR injection moulded parts. Equipped with a 64-cavity mould from ACH Solution and a Multilift Select16. This turnkey system produces around 9,200 LSR “Secure Seal” closures per hour to food approval compliance standards. These closures are used to seal the needles and syringes used in medical technology. The linear robotic system removes the injection-moulded parts, each weighing 0.5 grams, and sets them down separately in their respective mould cavities. Another drawer system is used to store parts for quality assurance.

(More information, see press release “Allrounder Trend 2026”)

Silicone mould inserts for small quantities

On a vertical Allrounder 375 V with manual workstation, Arburg and its partner Siocast are using toy figures and bottle openers to illustrate how plastic parts can be produced economically in small quantities. Numbers range from a few dozen to several thousand. This is achieved by combining 3D printing, silicone mould making and injection moulding. First of all, the desired digital model is designed. Using the associated CAD data, 3D printers begin by producing master models from synthetic resin and runners. In the next step, these are embedded in HTV silicone and vulcanised. What remains is the final mould. As a rule, several moulds are used. These are inserted successively into the injection moulding machine in turn so that the plastic figure can be injection moulded piece by piece.

Images

We will be happy to provide you with current photos from the Technology Days 2026 until the middle of calendar week 12 via the photo downloads and on request.

Photo download:

https://media.arburg.com/web/d18f08f0490a82ed/pr-photos-technology-days-2026/

Press

File: 03 ARBURG press release Silicone Technology Days 2026_en_GB.docx

Characters: 4,680

Words: 71^5

This and other press releases are available for download from our website at www.arburg.com/de/presse/ (www.arburg.com/en/press/)

Contact

ARBURG GmbH + Co KG

Press office

Susanne Palm

Dr Bettina Keck

Postfach 1109

72286 Lossburg, Germany

Tel.: +49 (0)7446 33-3463

Tel.: +49 (0)7446 33-3259

presse_service@arburg.com

About Arburg

Founded in 1923, the German family-owned company is one of the world's leading manufacturers of injection moulding machines. AMKmotion, a manufacturer of electric drive technology, also belongs to the Arburg family.

The portfolio for plastics processing encompasses Allrounder injection moulding machines, robot systems as well as customer- and industry-specific turnkey solutions. It also includes digital products and services. Arburg is a pioneer in the plastics industry when it comes to energy and production efficiency, digitalisation and sustainability. Arburg machines are used to manufacture plastic products for the mobility, packaging, electronics, medical, construction and equipment engineering and leisure industries, for example.

The company headquarters are located in Lossburg, Germany. In addition, Arburg has own organisations in 27 countries at 37 locations and is represented in over 100 countries together with trading partners. Of a total of roughly 3,400 employees, about 2,800 work in Germany, with another 600 employees based in Arburg's organisations around the world.

Arburg is certified to ISO 9001 (Quality), ISO 14001 (Environment), ISO 27001 (Information Security), ISO 29993 (Education and Training) and ISO 50001 (Energy).

Further information: www.arburg.com, www.amk-motion.com.